Firmware Upgrade

Company programming software will not recognize TD-H3 unless you upgrade the firmware

(Do not turn on the radio untill step6)

Step1 Install Firmware Upgrade Software

Step2 Open IAP_3773_3782

Step3 Connect your radio with a programming cable, choice corresponding port.

Step4 Click [OpenFile] and then select the firmware upgrade file

Step5 Press the upper part of the cable kenwood jack through out the upgrade process.

Step6 Click [Start] and then Press PPT+ turn on your radio

*There will be a progress bar to indicate the progress of the software when the upgrade starts correctly.*

Q1: What should i do if there are not progress bar after I turn on my radio

A1: Hold the plug of the programming cable tightly during the upgrade and do it again, never loose the grip.

Q2: What should i do if the upgrade fail and the radio cant turn on again.

A2:Because the upgrade process may be interrupted, resulting in an incomplete version upgrade of the radio. You need to repeat the upgrade steps again to completely accomplish the upgrade.

More question please email suppport@tidradio.com